# Industry project

Power load Balancing

Group name: The power rangers

Members: Casper, Han, Pleun, Toon, Ian

Problem description:	2

Project goal:	3

Requirements	3

System overview	3

Sprint planning	3

## Problem description:

With the growing number of electric vehicles (EVs) in workplaces, office buildings face increasing pressure on their electrical infrastructure. Many existing power connections are limited in capacity and were not originally designed to handle the additional demand from multiple EV chargers. When several vehicles charge simultaneously, the total energy consumption can easily exceed the building’s power limit, leading to potential overloads, inefficiencies, and increased energy costs. Additionally, if power is distributed without coordination, some vehicles may charge faster than others, creating unfairness and dissatisfaction among users. The combination of limited power capacity, varying demand, and fluctuating electricity prices makes uncoordinated EV charging both inefficient and potentially costly.

## Project goal:

The goal of this project is to develop a prototype of a system that can simulate and analyse power load distribution in a building with charging stations for electric vehicles. The project aims to explore strategies for efficient and fair allocation of the available electrical power between the buildings own consumption of electricity and the charging stations. Our solution should demonstrate how load balancing can improve energy efficiency while maintaining safe operating conditions within the 100 kW limit. We provide a fair basis for EV charging management in similar real world applications.

## Requirements

### functional

- The system operates in a decentralized manner. This avoids problems when a charging post fails.

- At least 3 modes of power distribution

- Has a dashboard to display the current state of the system

  - Current building consumption

  - Solar generation

  - Available power capacity

  - Charging post status and its assigned power

- The system ensures that all connected charging stations receive only as much power as that is available in the buildings capacity

- The main office has priority over charging stations, only the leftover power is used for charging vehicles.

- The system will manage at least four charging stations

- The system tracks the current building energy consumption in kW (simulated)

- The system tracks the current solar power that is being produced and adds it to the available capacity

- The system includes fuse protection logic, if total power consumption exceeds 100kW it triggers an alert and automatically adjust allocations to stay within safe limits.

- The system makes use of a reliable communication mechanism to exchange data between the building, solar generation unit, and charging posts.

- The system detects and responds to changes such as reduced solar output (e.g., cloudy conditions) by adjusting the power allocation accordingly.

### Non-functional:

- The charging posts will not charge faster than 11 KW (!)

- The prototype will simulate a threshold of 100 KW that represents the buildings maximum supply.

- The dashboard will update every 5 minutes

- The System will be able to handle changes to available power depending on solar power output.

- The system will be able to handle disconnections in communication between charging stations, as well as with the building.

- The system will be able to expand and handle more charging stations in the future

# Power Load Analysis and Distribution System - Use Cases

## Use Case 1: Dynamic Load Balancing During Cloudy Day

| Name | Dynamic Load Balancing During Cloudy Day |
| --- | --- |
| Description / Objective | Enable each charging station to independently calculate and allocate power based on available capacity during low solar input, ensuring fair distribution and system stability. |
| Preconditions | user authentication and scheduling system are active. Solar input = 5 kW, Grid = 100kW, building = 85kW, two charging stations active, system in dynamic mode. |
| Actors | EV owner, the system, EV charging controller, facility manager. |
| Main Flow | - Charging Station 1 detects a new car connection. - Building ESP32 broadcasts available power - Charging Station 1 receives the broadcasted available power. - Charging Station 1 syncs with other stations to determine the number of active charging sessions (2). - Charging Station 1 calculates its share - Charging Station 1 begins charging at 10 kW. - Charging Station 2 independently performs the same calculation and continues charging at 10 kW. |
| Alternate Flow | - A third car plugs in at 14:15 → All stations recalculate and adjust to 6.67 kW each. - Solar input drops to 0 kW → Available power becomes 15 kW → Each station adjusts to 7.5 kW. - Charging Station 2 disconnects → Charging Station 1 recalculates and receives full 15 kW. |
| Postcondition | EVs are charged efficiently without exceeding building power limits or causing load imbalance. Both active charging stations are charging at 10 kW; Total system consumption remains within the 100 kW limit; No central controller was involved, each station acted autonomously. |

## Use Case 2: First-Come-First-Serve Charging During Peak Office Hours

| Name | First-Come-First-Serve Charging During Peak Office Hours |
| --- | --- |
| Description / Objective | Ensure charging stations prioritize cars based on arrival time while staying within the building’s power limit. |
| Preconditions | Four charging stations are active, system is in FCFS mode. Grid capacity = 100 kW, Building load = 80 kW, Solar input = 10 kW, Available power = 30 kW |
| Actors | EV owner, the system, EV charging controller, facility manager. |
| Main Flow | - Charging Station 1 and 2 detect car connections first. - Building ESP32 broadcasts 30 kW available power. - Each station independently checks its arrival order. - Station 1 and 2 allocate 11 kW each. - Station 3 receives remaining 8 kW. - Station 4 receives 0 kW and enters wait state. |
| Alternate Flow | - Station 1 finishes charging → Station 4 recalculates and begins charging - Solar input drops --> all stations recalculate based on reduced available power |
| Postcondition | EVs are charged efficiently without exceeding building power limits or causing load imbalance. Total charging load = 30kW; Charging priority is based on arrival time. |

## Use Case 3: Director Vehicle Charging with Priority Mode Enabled

| Name | Director Vehicle Charging with Priority Mode Enabled |
| --- | --- |
| Description / Objective | Ensure director vehicles receive full charging speed while remaining power is fairly distributed among other cars. |
| Preconditions | Four charging stations are active, system is in directors mode. Grid capacity = 100 kW, Building load = 30 kW, Solar input = 0 kW, Available power = 70 kW, directors are identified by priority tag |
| Actors | EV owner, the system, EV charging controller, facility manager. |
| Main Flow | - Building ESP32 broadcasts 70 kW available power. - Each station receives broadcast and syncs with peers. - Director stations allocate 11 kW each (22 kW total). - Remaining 48 kW is divided between regular cars (24 kW each). - All stations begin charging independently. |
| Alternate Flow | - A third director car connects → Regular cars recalculate and receive reduced power. - One director car disconnects → Regular cars receive increased share. |
| Postcondition | EVs are charged efficiently without exceeding building power limits or causing load imbalance. Directors cars receive full speed charging. Regular cars receive fair share of remaining power. |

Use Case 4: Power Outage Detection and Response

| Name | Power Outage Detection and Response |
| --- | --- |
| Description / Objective | Detect partial or full building power outages and initiate predefined response protocols to maintain critical operations. |
| Preconditions | the system and backup power systems (e.g., UPS or generator) are installed and configured; critical loads are tagged in the system. |
| Actors | the system, power sensors, maintenance staff, facility manager. |
| Main Flow | 1. the system continuously monitors power flow across circuits. 2. A power drop or outage is detected in a section of the network. 3. the system activates backup power to supply critical systems (e.g., emergency lighting, servers). 4. Notifications are sent to maintenance staff for manual intervention. |
| Alternate Flow | - If backup system fails to start, the system triggers emergency shutdown of non-critical loads. - If outage is partial, the system reroutes available power to priority circuits. |
| Postcondition | Critical operations remain functional, and outage impact is minimized until full power is restored. |

## Use Case 5: Energy Consumption Analytics and Reporting

| Name | Energy Consumption Analytics and Reporting |
| --- | --- |
| Description / Objective | Provide real-time and historical analytics on energy consumption patterns, including solar utilization, EV charging demand, and building load trends. |
| Preconditions | the system is collecting data from all subsystems; analytics dashboard and database are functional. |
| Actors | the system, building manager, sustainability officer, data analyst. |
| Main Flow | 1. the system aggregates energy data from building loads, solar panels, and chargers. 2. System generates daily, weekly, and monthly reports. 3. Manager reviews trends and identifies optimization opportunities. 4. Reports are exported for sustainability tracking or compliance documentation. |
| Alternate Flow | - If data connection to sensors fails, system uses cached or last known values. - User can request on-demand reports or custom visualizations. |
| Postcondition | Building stakeholders gain actionable insights for optimizing energy efficiency and planning upgrades. |

## System overview

This system simulates a decentralized power load balancing solution for an office building with electric vehicle charging stations. The Building ESP32 calculates available power based on grid capacity, office consumption, and solar input, then broadcasts this the left-over power to all Charging Station ESP32s. Each station independently determines how much power it can use by syncing with other stations to detect how many are online and applying the selected load balancing mode (Static, Dynamic, First-Come-First-Serve, or Directors Parking). Charging stations send their status back to the dashboard ESP32, which displays the data coming from the charging stations, enabling real-time monitoring. Peer-to-peer communication ensures robustness and scalability without centralized control.

### Load balancing modes overview

This system supports four load balancing modes to manage how available power is distributed across electric vehicle charging stations. Each mode ensures the total consumption never exceeds the building’s 100 kW limit, while adapting to real-time conditions like car presence and solar input.

1. Static Load Balancing

Power is evenly divided among all charging stations, regardless of whether a car is connected. This mode is simple but may waste power if some stations are idle.

2. Dynamic Load Balancing

Available power is equally shared only among stations with active charging sessions. This ensures efficient use of energy and adapts to the number of connected vehicles.

3. First-Come-First-Serve (FCFS)

Cars are charged in the order they arrive. Each gets up to 11 kW until capacity runs out. Later arrivals may receive reduced power or wait until earlier cars finish charging.

4. Directors Parking

Designated director vehicles always receive 11 kW. Remaining power is distributed among other cars using dynamic allocation. This mode prioritizes specific users while maintaining fairness for others.

Each station independently calculates its share based on the selected mode, number of active stations, and available power broadcasted by the building controller. This decentralized logic ensures robustness and scalability.

To demonstrate the distribution of power we made an interactive diagram:

https://paint-fake-83815868.figma.site/

## Sprint planning

Planning of each sprint (in which week, deliverables, deadlines)

Sprint 1 – Concept design (30-09-2025 – 07-10-2025)

Sprint 2 – Design update (07-10-2025 – 28-10-2025)

- Project description and design.

Sprint 3 – Talking skeleton (28-10-2025 – 25-11-2025)

- Communication established between 3 or more of the modules.

- Communication protocol implemented.

Sprint 4 – Final design, testing and integration (25-11-2025 – 16-12-2025)

Sprint 5 – Final delivery (16-12-2025 – 20-01-2026)