# Power Load Analysis and Distribution System - Use Cases

## Use Case 1: Dynamic Load Balancing During Cloudy Day

| Name | Dynamic Load Balancing During Cloudy Day |
| --- | --- |
| Description / Objective | Enable each charging station to independently calculate and allocate power based on available capacity during low solar input, ensuring fair distribution and system stability. |
| Preconditions | user authentication and scheduling system are active. Solar input = 5 kW, Grid = 100kW, building = 85kW, two charging stations active, system in dynamic mode. |
| Actors | EV owner, the system, EV charging controller, facility manager. |
| Main Flow | - Charging Station 1 detects a new car connection. - Building ESP32 broadcasts available power - Charging Station 1 receives the broadcasted available power. - Charging Station 1 syncs with other stations to determine the number of active charging sessions (2). - Charging Station 1 calculates its share - Charging Station 1 begins charging at 10 kW. - Charging Station 2 independently performs the same calculation and continues charging at 10 kW. |
| Alternate Flow | - A third car plugs in at 14:15 → All stations recalculate and adjust to 6.67 kW each. - Solar input drops to 0 kW → Available power becomes 15 kW → Each station adjusts to 7.5 kW. - Charging Station 2 disconnects → Charging Station 1 recalculates and receives full 15 kW. |
| Postcondition | EVs are charged efficiently without exceeding building power limits or causing load imbalance. Both active charging stations are charging at 10 kW; Total system consumption remains within the 100 kW limit; No central controller was involved, each station acted autonomously. |

## Use Case 2: First-Come-First-Serve Charging During Peak Office Hours

| Name | First-Come-First-Serve Charging During Peak Office Hours |
| --- | --- |
| Description / Objective | Ensure charging stations prioritize cars based on arrival time while staying within the building’s power limit. |
| Preconditions | Four charging stations are active, system is in FCFS mode. Grid capacity = 100 kW, Building load = 80 kW, Solar input = 10 kW, Available power = 30 kW |
| Actors | EV owner, the system, EV charging controller, facility manager. |
| Main Flow | - Charging Station 1 and 2 detect car connections first. - Building ESP32 broadcasts 30 kW available power. - Each station independently checks its arrival order. - Station 1 and 2 allocate 11 kW each. - Station 3 receives remaining 8 kW. - Station 4 receives 0 kW and enters wait state. |
| Alternate Flow | - Station 1 finishes charging → Station 4 recalculates and begins charging - Solar input drops --> all stations recalculate based on reduced available power |
| Postcondition | EVs are charged efficiently without exceeding building power limits or causing load imbalance. Total charging load = 30kW; Charging priority is based on arrival time. |

## Use Case 3: Director Vehicle Charging with Priority Mode Enabled

| Name | Director Vehicle Charging with Priority Mode Enabled |
| --- | --- |
| Description / Objective | Ensure director vehicles receive full charging speed while remaining power is fairly distributed among other cars. |
| Preconditions | Four charging stations are active, system is in directors mode. Grid capacity = 100 kW, Building load = 30 kW, Solar input = 0 kW, Available power = 70 kW, directors are identified by priority tag |
| Actors | EV owner, the system, EV charging controller, facility manager. |
| Main Flow | - Building ESP32 broadcasts 70 kW available power. - Each station receives broadcast and syncs with peers. - Director stations allocate 11 kW each (22 kW total). - Remaining 48 kW is divided between regular cars (24 kW each). - All stations begin charging independently. |
| Alternate Flow | - A third director car connects → Regular cars recalculate and receive reduced power. - One director car disconnects → Regular cars receive increased share. |
| Postcondition | EVs are charged efficiently without exceeding building power limits or causing load imbalance. Directors cars receive full speed charging. Regular cars receive fair share of remaining power. |

Use Case 4: Power Outage Detection and Response

| Name | Power Outage Detection and Response |
| --- | --- |
| Description / Objective | Detect partial or full building power outages and initiate predefined response protocols to maintain critical operations. |
| Preconditions | the system and backup power systems (e.g., UPS or generator) are installed and configured; critical loads are tagged in the system. |
| Actors | the system, power sensors, maintenance staff, facility manager. |
| Main Flow | 1. the system continuously monitors power flow across circuits. 2. A power drop or outage is detected in a section of the network. 3. the system activates backup power to supply critical systems (e.g., emergency lighting, servers). 4. Notifications are sent to maintenance staff for manual intervention. |
| Alternate Flow | - If backup system fails to start, the system triggers emergency shutdown of non-critical loads. - If outage is partial, the system reroutes available power to priority circuits. |
| Postcondition | Critical operations remain functional, and outage impact is minimized until full power is restored. |

## Use Case 5: Energy Consumption Analytics and Reporting

| Name | Energy Consumption Analytics and Reporting |
| --- | --- |
| Description / Objective | Provide real-time and historical analytics on energy consumption patterns, including solar utilization, EV charging demand, and building load trends. |
| Preconditions | the system is collecting data from all subsystems; analytics dashboard and database are functional. |
| Actors | the system, building manager, sustainability officer, data analyst. |
| Main Flow | 1. the system aggregates energy data from building loads, solar panels, and chargers. 2. System generates daily, weekly, and monthly reports. 3. Manager reviews trends and identifies optimization opportunities. 4. Reports are exported for sustainability tracking or compliance documentation. |
| Alternate Flow | - If data connection to sensors fails, system uses cached or last known values. - User can request on-demand reports or custom visualizations. |
| Postcondition | Building stakeholders gain actionable insights for optimizing energy efficiency and planning upgrades. |