# Industry project

Power load Balancing

Group name: The power rangers

Members: Casper, Han, Pleun, Toon, Ian

Requirement:	2

functional	3

Non-functional:	3

Task decomposition:	3

Four concrete Task flows:	4

Power task flow:	4

Communication task flow:	4

Research questions:	5

Extra:	5

Project setup	5

Power tracking and statistics	5

Charging station management	5

Building consumption simulation	6

Balancing modes	6

Fuse protection logic	6

Dashboard	6

# Requirement:

## functional

- The system operates in a decentralized manner. This avoids problems when a charging post fails.

- At least 3 modes of power distribution

- Has a dashboard to display the current state of the system

  - Current building consumption

  - Solar generation

  - Available power capacity

  - Charging post status and its assigned power

- The system ensures that all connected charging stations receive only as much power as that is available in the buildings capacity

- The main office has priority over charging stations, only the leftover power is used for charging vehicles.

- The system will manage at least four charging stations

- The system tracks the current building energy consumption in kW (simulated)

- The system tracks the current solar power that is being produced and adds it to the available capacity

- The system includes fuse protection logic, if total power consumption exceeds 100kW it triggers an alert and automatically adjust allocations to stay within safe limits.

- The system makes use of a reliable communication mechanism to exchange data between the building, solar generation unit, and charging posts.

- The system detects and responds to changes such as reduced solar output (e.g., cloudy conditions) by adjusting the power allocation accordingly.

## Non-functional:

- The charging posts will not charge faster than 11 KW (!)

- The prototype will simulate a threshold of 100 KW that represents the buildings maximum supply.

- The dashboard will update every 5 minutes

- The System will be able to handle changes to available power depending on solar power output.

- The system will be able to handle disconnections in communication between charging stations, as well as with the building.

- The system will be able to expand and handle more charging stations in the future

# Task decomposition:

| Sub-system | description |
| --- | --- |
| GUI | The system that displays all the information provided and the system that handles all the interaction with the user |
| building | Requires a fixed amount of energy and is first in priority |
| Solar power management | Generates a varying amount of power first in priority when providing power |
| Power-grid management system | Provides the rest of the needed power |
| Power station 1 | Communicates with the other power stations and divides the power |
| Power station 2 | Communicates with the other power stations and divides the power |
| Power station 3 | Communicates with the other power stations and divides the power |
| Power station 4 | Communicates with the other power stations and divides the power |

# Four concrete Task flows:

## Power task flow:

.

## Communication task flow:

# Research questions:

How do you know if a car/device is fully charged?

How does the chargers know which chargers doesn’t work?

How does the building affect the charging of a car/devices?

What if the solar energy gives more power than it needs?

What UI do we want? Serial, web-based, other?

# Extra:

## Project setup

  - Git

  - VS studio code ready

  - Define roles and responsibilities

## Power tracking and statistics

  - Simulate building consumption

  - Simulate solar energy production

  - Track the available power

  - Store and update power

## Charging station management

  - System for four charging stations

  - Track connection status (connected and disconnected)

  - Track each charger's current power usage

  - Limits (no more than 11 kW per station)

Building consumption simulation

- Power source simulation

- Simulate consistent power-drain from power source

- Simulate additional power generation from solar panels

- Simulate different amount of energy consumption depending on time of day

## Balancing modes

  - Logic to switch between modes

  - Static load balancing

  - Dynamic load balancing

  - First come first serve

  - Directors mode

## Fuse protection logic

  - Monitor total consumption

  - Trigger alert/adjust charging when usage nears 100kW

  - Make sure that the system never exceeds total capacity

## Dashboard

  - Display the current building consumption and solar power

  - Show power per charging station

  - Show alerts

  - Switching modes?

  - Refresh every 5 minutes